# NIGHTJAR–VISIONSHIELD-X

## Multimodal Perimeter Activity Detection System

Software Architecture & Implementation Planning Document

Version 1.0 - Complete Software Design Specification

## TABLE OF CONTENTS

1. Executive Summary

2. System Architecture Overview

3. Software Components & Modules

4. Models to be Created

5. Algorithm Specifications

6. Data Flow & Integration

7. Implementation Stack

8. Testing Strategy

9. Deployment & Monitoring

## 1. EXECUTIVE SUMMARY

NIGHTJAR–VISIONSHIELD-X is a compact, edge-AI multimodal perimeter activity detection system designed to operate on low-cost Raspberry Pi hardware. The system combines temporal RGB image analysis with low-resolution thermal sensing to detect and confirm activity events in monitored zones.

Key Objectives:

Reduce false alarms through multi-sensor evidence accumulation

Operate effectively under reduced visibility conditions (fog, rain, low light)

Maintain low hardware and computational costs

Enable local edge processing without cloud dependency

Provide real-time monitoring dashboard with event logging

## 2. SYSTEM ARCHITECTURE OVERVIEW

### 2.1 High-Level Architecture

The system consists of two primary data acquisition layers feeding into a multi-stage processing pipeline:

| Layer | Components |
| --- | --- |
| Acquisition | RGB Camera, MLX90640 Thermal Sensor, Raspberry Pi 4 |
| Processing | Frame capture, temporal analysis, object detection, thermal processing |
| Fusion | Evidence-level fusion, event confirmation, visibility estimation |
| Output | Event logging, notifications, dashboard visualization, recordings |

### 2.2 Processing Pipeline Flow

Input → Temporal Analysis → Perimeter Check → Object Detection → Tracking → Visibility Analysis + Thermal Analysis → Evidence Fusion → Event Confirmation → Output

## 3. SOFTWARE COMPONENTS & MODULES

### 3.1 Core Modules

Module 1: Frame Capture & Buffer Management

Responsibility: Continuous RGB frame acquisition from camera at configured frame rate (15-30 FPS)

Key Functions:

• Initialize camera device (OpenCV VideoCapture)

• Maintain 5-frame circular buffer for temporal analysis

• Generate reference frame (running average or manual capture)

• Handle frame timestamps and sequencing

Module 2: Temporal Change Detection

Responsibility: Detect persistent scene changes across 5-second window

Key Functions:

• Compare current frame against reference frame (absolute difference)

• Apply Gaussian blur and morphological operations to reduce noise

• Generate binary change mask with configurable threshold

• Track persistence of changes across temporal buffer

• Compute temporal change confidence score

Module 3: Perimeter Definition & Checking

Responsibility: Restrict analysis to user-defined monitoring zones

Key Functions:

• Accept polygonal region definition from user interface

• Store polygon vertices and create binary mask

• Test detected regions against perimeter constraint (point-in-polygon test)

• Generate perimeter confidence score (100% if inside, 0% if outside)

• Support multiple named zones/regions

Module 4: Lightweight Object Detection

Responsibility: Detect potential activity objects (people, vehicles) in RGB frames

Key Functions:

• Load pre-trained lightweight model (YOLOv4-Tiny or similar)

• Perform inference on current frame at reduced resolution (320x320 or 416x416)

• Filter detections by confidence threshold (configurable, typically 0.4-0.6)

• Extract bounding boxes and associated confidence scores

• Generate RGB detection confidence metric

Module 5: Object Tracking

Responsibility: Track detected objects across frames to establish trajectory

Key Functions:

• Implement Centroid Tracking or Deep SORT for stable object identity

• Match detections from current frame to previous track states

• Compute trajectory (velocity, direction) from historical positions

• Maintain track ID and temporal history

• Output tracking confidence based on detection consistency

Module 6: Visibility Quality Estimation

Responsibility: Estimate RGB image quality and visibility conditions

Key Functions:

• Compute image sharpness (Laplacian variance)

• Compute image brightness (mean pixel intensity)

• Compute image contrast (standard deviation)

• Compute edge density (Canny edge detector)

• Aggregate metrics into single visibility score (0-100)

• Classify visibility state: Good, Reduced, Poor

Module 7: Thermal Analysis

Responsibility: Process thermal sensor data and detect thermal anomalies

Key Functions:

• Read temperature matrix from MLX90640 (32x24 array)

• Compute background thermal statistics (mean, std dev)

• Detect thermal hotspots above ambient + threshold

• Map thermal region to RGB frame via coordinate transformation

• Generate thermal evidence score

Module 8: Evidence-Level Fusion Engine

Responsibility: Combine evidence from multiple sources into unified confidence metric

Key Functions:

• Aggregate confidence scores: RGB detection, temporal change, perimeter, thermal, visibility

• Apply configurable weights to each evidence source (w_R + w_T + w_C + w_P = 1)

• Compute weighted fusion score: C_F = w_R*C_R + w_T*C_T + w_C*C_C + w_P*C_P

• Apply visibility multiplier to reduce confidence in poor visibility

• Generate debug/reasoning string for event log

Module 9: Event Confirmation Logic

Responsibility: Implement two-signal confirmation to minimize false positives

Key Functions:

• Classify events as Candidate (1 evidence source) or Confirmed (2+ sources)

• Require both RGB and Thermal evidence for full confirmation

• Allow Temporal + RGB as alternative confirmation path

• Implement event state machine (New → Candidate → Confirmed → Closed)

• Generate event metadata and unique event ID

Module 10: Event Recording & Archival

Responsibility: Capture and store event evidence for later review

Key Functions:

• Save pre-event RGB frame (1 frame before detection)

• Save event RGB frame (detection moment)

• Save temporal sequence (all 5 buffered frames)

• Save thermal array as NumPy (.npy) and visualization (.png)

• Generate event.json with full metadata (timestamp, scores, zone, etc)

• Implement configurable retention policy (disk space management)

Module 11: Notification System

Responsibility: Alert operators to confirmed events

Key Functions:

• Generate notification payload with event details

• Support multiple notification channels (email, HTTP webhook, local alert)

• Include thumbnail, timestamp, zone, confidence

• Implement notification rate-limiting to prevent alert fatigue

• Log all notifications to event database

Module 12: Real-Time Dashboard

Responsibility: Provide operator interface for monitoring and configuration

Key Functions:

• Display live RGB camera feed with detection overlays

• Display thermal array as heatmap with temperature scale

• Show perimeter zone polygon overlay on RGB

• Display real-time evidence status (RGB check, Thermal check, Perimeter check)

• Show visibility quality metric

• Display sensor health status (camera, thermal, processor load)

• Provide event history log with sortable/filterable interface

• Enable configuration controls (threshold adjustment, zone editing)

## 4. MODELS TO BE CREATED

### 4.1 Object Detection Model

Model Type: Pre-trained lightweight CNN (not to be trained, to be integrated)

Purpose: Detect people, vehicles, and other objects of interest in RGB frames

Recommended Models:

• YOLOv4-Tiny (33-50 MB, ~40 FPS on RPi4)

• MobileNetV2 + SSD (lightweight, ~25 FPS)

• TinyYOLOv3 (compact, balanced accuracy/speed)

Input: RGB frame at 320x320 or 416x416 resolution

Output: Bounding boxes with class labels and confidence scores (person, vehicle, etc)

Framework: ONNX or TensorFlow Lite for edge inference

Integration Method: Load pre-trained weights, run inference on every frame, post-process detections

### 4.2 Tracking Model

Model Type: Stateful tracking algorithm (not neural network)

Purpose: Associate detections across frames to establish object identity and trajectory

Recommended Approaches:

• Centroid Tracking (simplest, lightweight)

- Match bounding boxes by centroid distance

- Assign stable IDs to tracked objects

• Hungarian Algorithm (IoU-based matching)

- More robust to occlusion and crowding

• Kalman Filter + Hungarian (predictive tracking)

- Predict object position in next frame

- Reduce false associations

Input: Detection bounding boxes from current frame and previous frame

Output: Track IDs, updated bounding boxes, trajectory information

### 4.3 Change Detection Model

Model Type: Optical flow or background subtraction (no neural network required)

Purpose: Detect persistent scene changes over 5-second temporal window

Algorithm:

1. Maintain stable reference frame (updated periodically or on user command)

2. Compute absolute difference: D = |I_current - I_reference|

3. Convert to grayscale and apply threshold (T=30, configurable)

4. Morphological filtering (erosion + dilation) to reduce noise

5. Track persistence of changes across temporal buffer (5 frames)

6. Compute change confidence: fraction of buffer frames showing change

Input: Current RGB frame and reference frame

Output: Binary change mask, change confidence score, region of change

### 4.4 Thermal Anomaly Detection Model

Model Type: Statistical anomaly detection (no machine learning model)

Purpose: Detect thermal hotspots indicating living objects (people, animals)

Algorithm:

1. Read 32x24 temperature matrix from MLX90640 sensor

2. Compute background thermal statistics (mean T_bg, std T_bg)

3. Identify pixels exceeding threshold: T_pixel > T_bg + 3*sigma

4. Cluster thermal pixels into connected components

5. Validate clusters by spatial coherence and size

6. Map thermal cluster to RGB frame coordinate space

7. Compute thermal confidence: cluster size/intensity score

Input: Temperature matrix from MLX90640 (updated ~8 Hz)

Output: Thermal anomaly regions, confidence score, mapped bounding box in RGB space

### 4.5 Visibility Quality Model

Model Type: Image quality assessment (computer vision metrics)

Purpose: Quantify RGB image quality to adjust detector confidence in poor visibility

Metrics to Compute:

• Sharpness: Laplacian variance (higher = sharper)

• Brightness: Mean pixel intensity normalized [0-100]

• Contrast: Standard deviation of pixel intensities [0-100]

• Edge Density: Count of edge pixels via Canny detector [0-100]

Aggregation:

V_score = 0.3*Sharpness + 0.2*Brightness + 0.3*Contrast + 0.2*EdgeDensity

Output: Visibility score [0-100], Visibility state (Good/Reduced/Poor)

## 5. ALGORITHM SPECIFICATIONS

### 5.1 Temporal RGB Change Detection Algorithm

Input:

I_current: current RGB frame

I_reference: stable reference frame

T: change detection threshold (default 30)

Algorithm:

1. Compute absolute pixel difference: D(x,y) = |I_current(x,y) - I_reference(x,y)|

2. Convert D to grayscale (if color image)

3. Apply Gaussian blur (kernel 5x5, sigma 1.0) to reduce noise

4. Apply binary threshold: M(x,y) = 1 if D(x,y) > T else 0

5. Morphological opening (erode 3x3, then dilate 3x3) to remove small noise

6. Find contours of changed regions

7. Filter contours by minimum area (e.g., 500 pixels)

8. Add non-zero pixel count to temporal buffer (5-frame history)

9. Compute persistence: (frames with change) / 5 = confidence [0-1]

Output:

M: binary change mask

regions: list of changed regions (contours)

C_C: temporal change confidence [0-1]

### 5.2 Perimeter Checking Algorithm (Point-in-Polygon)

Input:

polygon: array of (x,y) vertices defining monitored zone

region: bounding box or contour of detected activity

Algorithm:

1. Compute centroid of region: (cx, cy) = center of bounding box

2. Use Ray Casting Algorithm: cast ray from (cx, cy) to infinity

3. Count intersections of ray with polygon edges

4. If odd number of intersections: point is inside (C_P = 1.0)

5. If even number of intersections: point is outside (C_P = 0.0)

Output:

C_P: perimeter confidence (0 or 1)

zone_name: assigned zone if inside

### 5.3 Evidence Fusion Algorithm

Input:

C_R: RGB detection confidence [0-1]

C_T: thermal evidence confidence [0-1]

C_C: temporal change confidence [0-1]

C_P: perimeter confidence [0-1]

V_score: visibility score [0-100]

w_R, w_T, w_C, w_P: fusion weights (sum to 1.0)

Algorithm:

1. Normalize visibility score: V_norm = V_score / 100

2. Compute base fusion: C_F = w_R*C_R + w_T*C_T + w_C*C_C + w_P*C_P

3. Apply visibility multiplier: C_F_adjusted = C_F * V_norm

4. Clamp result to [0-1]

5. Count how many evidence sources are above threshold (0.5)

Output:

C_F: fused confidence score [0-1]

evidence_count: number of sources supporting event

reasoning: human-readable explanation of fusion

### 5.4 Event Confirmation State Machine

States:

• NEW: Activity detected but not yet classified

• CANDIDATE: Single evidence source above threshold

• CONFIRMED: Two or more independent evidence sources satisfy criteria

• CLOSED: Event processed, recorded, and notification sent

Transitions:

NEW → CANDIDATE: If C_R > 0.5 OR C_T > 0.5 OR C_C > 0.5

CANDIDATE → CONFIRMED: If (C_R > 0.5 AND C_T > 0.5) OR (C_R > 0.5 AND C_C > 0.5) OR (C_T > 0.5 AND C_C > 0.5)

CONFIRMED → CLOSED: After notification sent and event recorded

## 6. DATA FLOW & INTEGRATION

### 6.1 End-to-End Data Flow

Step 1: Frame Acquisition

RGB camera (OpenCV) → YUV frame buffer (circular, 5 frames)

Step 2: Temporal Analysis

Current frame + Reference frame → Change detection → Temporal persistence → C_C [0-1]

Step 3: Perimeter Check

Change regions → Point-in-polygon test → C_P (0 or 1)

Step 4: Object Detection (in parallel)

Current frame → YOLO inference → Detections + confidence → C_R [0-1]

Step 5: Tracking (in parallel)

Detections + Previous tracks → Matching → Track IDs + trajectory

Step 6: Visibility Analysis (in parallel)

Current frame → Image metrics → V_score [0-100]

Step 7: Thermal Analysis (in parallel, ~8 Hz)

MLX90640 → Temperature matrix → Anomaly detection → C_T [0-1] + region

Step 8: Evidence Fusion

C_R, C_T, C_C, C_P, V_score → Weighted fusion → C_F [0-1]

Step 9: Event Confirmation

C_F + evidence_count → State machine → CONFIRMED/CANDIDATE/CLOSED

Step 10: Event Recording

If CONFIRMED → Save RGB frames, thermal array, metadata → event.json

Step 11: Notification

If CONFIRMED → Send alert via configured channels (webhook, email, local)

Step 12: Dashboard Update

Event + RGB + Thermal + Scores → Real-time display + event log

### 6.2 Component Integration Points

| Component A | Component B | Data Exchanged |
| --- | --- | --- |
| Frame Capture | Temporal Change | RGB frames (circular buffer) |
| Temporal Change | Perimeter Check | Change regions, C_C confidence |
| Object Detection | Tracking | Bounding boxes, class labels, C_R |
| Tracking | Evidence Fusion | Track IDs, confidence, trajectory |
| Thermal Analysis | Evidence Fusion | Thermal region, C_T, mapped coordinates |
| Visibility Est. | Evidence Fusion | V_score [0-100], visibility state |
| Evidence Fusion | Event Confirmation | C_F, evidence_count, reasoning |
| Event Confirmation | Event Recording | Event status, metadata, images |
| Event Confirmation | Notification | Event type, timestamp, confidence |
| Event Recording | Dashboard | Event log, recorded images/thermal |

## 7. IMPLEMENTATION STACK

### 7.1 Software Dependencies

| Category | Library/Framework | Version/Notes |
| --- | --- | --- |
| OS | Linux (Raspberry Pi OS) | Bullseye or later |
| Language | Python 3 | 3.8 or later |
| Image Processing | OpenCV | 4.5+ (compiled for ARM) |
| Numerical Computing | NumPy | Latest stable |
| Deep Learning | ONNX Runtime OR TensorFlow Lite | For model inference |
| Thermal Sensor | MLX90640 Python driver | adafruit-circuitpython-mlx90640 |
| Web Framework | Flask or FastAPI | For dashboard REST API |
| Dashboard Frontend | HTML5 + JavaScript (Plotly or Three.js) | Real-time visualization |
| Database | SQLite | Local event logging |
| Notifications | requests library | For HTTP webhooks |

### 7.2 Project Structure

nightjar_visionshield/

├── main.py                        # Main entry point

├── config.yaml                    # Configuration file

├── requirements.txt               # Python dependencies

├── src/

│   ├── capture.py                # Frame capture module

│   ├── temporal_analysis.py      # Change detection module

│   ├── perimeter.py              # Perimeter checking module

│   ├── detector.py               # Object detection module

│   ├── tracker.py                # Object tracking module

│   ├── visibility.py             # Visibility estimation module

│   ├── thermal_analysis.py       # Thermal analysis module

│   ├── fusion.py                 # Evidence fusion engine

│   ├── event_manager.py          # Event confirmation logic

│   ├── recorder.py               # Event recording module

│   ├── notifier.py               # Notification system

│   └── utils.py                  # Utility functions

├── api/

│   ├── app.py                    # Flask/FastAPI application

│   ├── routes.py                 # API endpoints

│   └── models.py                 # API request/response models

├── dashboard/

│   ├── index.html                # Main dashboard page

│   ├── css/

│   ├── js/

│   └── templates/

├── models/                       # Pre-trained model weights

│   ├── yolov4_tiny.onnx

│   └── class_names.txt

├── events/                       # Event storage directory

│   ├── event_001/

│   │   ├── rgb_before.jpg

│   │   ├── rgb_event.jpg

│   │   ├── sequence/

│   │   ├── thermal.npy

│   │   └── event.json

├── database/

│   └── events.db                 # SQLite event database

├── logs/                         # System logs

└── tests/

├── test_temporal.py          # Unit tests for modules

├── test_detector.py

└── test_fusion.py

## 8. TESTING STRATEGY

### 8.1 Module-Level Testing

Each module must have unit tests verifying correctness:

| Module | Test Cases |
| --- | --- |
| Temporal Change | No change: confidence=0; Persistent change: confidence→1; Noise filtering |
| Perimeter Check | Point inside: C_P=1; Point outside: C_P=0; Edge cases (on boundary) |
| Detector | Correct detections; Confidence scores in [0-1]; NMS (non-max suppression) working |
| Tracker | Consistent IDs across frames; Handles occlusion; ID switches < 5% |
| Visibility | Sharp image: high score; Blurry image: low score; Range [0-100] |
| Thermal Analysis | Person detection in thermal; Temperature range validation |
| Fusion Engine | Weights sum to 1.0; Output in [0-1]; Visibility multiplier applied |
| Event Manager | State transitions correct; Two-signal confirmation enforced |

### 8.2 Integration Testing

Test Configuration A - RGB Only

• Enable only object detection + temporal change

• Disable thermal and perimeter constraints

• Measure: Precision, Recall, False Alarm Rate

Test Configuration B - RGB + Temporal

• Enable temporal change persistence

• Disable thermal and perimeter

• Measure: Reduction in false alarms vs. Config A

Test Configuration C - RGB + Perimeter

• Enable perimeter constraint

• Disable thermal

• Measure: Events outside zone correctly ignored

Test Configuration D - RGB + Thermal

• Enable thermal evidence

• Disable temporal persistence requirement

• Measure: Thermal false positives, RGB-thermal agreement

Test Configuration E - Complete System

• Enable all modules: temporal + perimeter + thermal + visibility + fusion

• Enforce two-signal confirmation

• Measure: Overall performance vs. baseline (Config A)

### 8.3 Environmental Test Conditions

Condition 1: Normal Daylight

Expected: Baseline performance, high visibility score

Condition 2: Low Illumination (Dusk/Night)

Expected: RGB confidence drops, thermal becomes primary evidence, visibility score < 50

Condition 3: Simulated Fog

Expected: Reduced RGB contrast, edge density drops, thermal maintains detection

Condition 4: Light Rain/Spray

Expected: Temporal change artifacts from raindrops, thermal helps discriminate

Condition 5: Background Movement (Vegetation, Shadows)

Expected: Temporal persistence filters out non-human motion, thermal provides confirmation

Condition 6: Partial Occlusion

Expected: Tracking maintains object identity, thermal may detect visible portion

Condition 7: Low-Contrast Object/Background

Expected: Object detection confidence drops, thermal supports if target is warm

## 9. DEPLOYMENT & MONITORING

### 9.1 Deployment Checklist

Pre-Deployment:

☐ All unit tests passing

☐ Integration tests completed for all configurations

☐ YOLO model quantized and tested on Raspberry Pi

☐ Thermal sensor calibration verified

☐ Camera focus and exposure settings optimized

☐ Config file with all thresholds documented

Deployment:

☐ Clone repository to Raspberry Pi

☐ Install dependencies: pip install -r requirements.txt

☐ Create events/ and logs/ directories

☐ Initialize SQLite database

☐ Start system: python main.py

☐ Verify dashboard accessible at http://localhost:5000

Post-Deployment Monitoring:

• Monitor CPU usage (target < 80%)

• Monitor memory usage (target < 500 MB)

• Track event frequency and false alarm rate

• Log performance metrics hourly

• Review event recordings weekly for accuracy

## CONCLUSION

This software architecture document specifies the complete design for NIGHTJAR–VISIONSHIELD-X, a multimodal edge-AI perimeter monitoring system. The system integrates 12 core modules performing temporal RGB analysis, low-resolution thermal sensing, evidence fusion, and event confirmation.

The implementation prioritizes:

• Low computational overhead for Raspberry Pi deployment

• Reduction of false alarms through multi-sensor evidence accumulation

• Robustness to environmental variation (fog, rain, low light)

• Local edge processing without cloud dependency

• Real-time operator visibility through comprehensive dashboard

Development follows a modular design allowing independent testing and incremental integration. The proposed experimental methodology compares progressive system configurations to quantify the benefit of each component.

Document Version: 1.0

Date: September 26, 2026

Status: Software Architecture Finalized